# Measurements and Uncertainties

### LAB MANUAL 04

| Name | Reg. no. | Report Marks / 10 | Viva Marks / 5 | Total/15 |
| --- | --- | --- | --- | --- |

#### EXPERIMENT NO. 04

##### Measurements and Uncertainties

Objectives

To learn to deal with measurement uncertainty, to measure, to find its effect on subsequent calculations and to correctly report confidence intervals in experimental results. To illustrate these general principles, in this experiment we will measure the mass density of objects, and calculate the uncertainty in their determinations.

##### Required Tools

Vernier calipers Ruler

Set of objects whose density will be measured

- Metal Sphere

- Ring

##### Theory

###### Significant figures

There are three rules on determining how many significant figures are in a number:

- Non-zero digits are always significant.

- Any zeros between two significant digits are significant.

- A final zero or trailing zeros in the decimal portion only are significant.

Rules for Rounding off

When rounding, examine the figure following (i.e., to the right of) the figure that is to be last. This figure you are examining is the first figure to be dropped.

- If it is less than 5, drop it and all the figures to the right of it.

- If it is more than 5, increase by 1 the number to be rounded, that is, the preceeding figure.

- If it is 5, round the number so that it will be even. Keep in mind that zero is considered to be even when rounding off.

Example #1 - Suppose you wish to round 62.5347 to four significant figures. Look at the fifth figure. It is a 4, a number less than 5. Therefore, you will simply drop every figure after the fourth, and the original number rounds off to 62.53.

Example #2 - Round 3.78721 to three significant figures. Look at the fourth figure. It is 7, a number greater than 5, so you round the original number up to 3.79.

Example #3 - Round 24.8514 to three significant figures. Look at the fourth figure. It is a 5, so now you must also look at the third figure. It is 8, an even number, so you simply drop the 5and the figures that follow it. The original number becomes 24.8.

Addition and Subtraction

In mathematical operations involving significant figures, the answer is reported in such a way that it reflects the reliability of the least precise operation. Let's state that another way: a chain is

no stronger than its weakest link. An answer is no more precise that the least precise number used to get the answer. Let's do it one more time: imagine a team race where you and your team must finish together. Who dictates the speed of the team? Of course, the slowest member of the team. Your answer cannot be more precise than the least precise measurement.

For addition and subtraction, look at the decimal portion (i.e., to the right of the decimal point) of

the numbers only. Here is what to do:

- Count the number of significant figures in the decimal portion of each number in the problem. (The digits to the left of the decimal place are not used to determine the number of decimal places in the final answer.)

- Add or subtract in the normal fashion.

- Round the answer to the least number of places in the decimal portion of any number in the problem.

Multiplication and Division

The following rule applies for multiplication and division:

The least number of significant figures in any number of the problem determines the number of significant figures in the answer.

This means you must know how to recognize significant figures in order to use this rule.

Example #1: 2.5 x 3.42.

The answer to this problem would be 8.6 (which was rounded from the calculator reading of 8.55). Why?

2.5 has two significant figures while 3.42 has three. Two significant figures are less precise than three, so the answer has two significant figures.

Example #2: How many significant figures will the answer to 3.10 x 4.520 have?

Three is the correct answer. 14.0 has three significant figures. All trailing zeros in the decimal portion are considered significant.

Another common error is for the student to think that 14 and 14.0 are the same thing. THEY ARE NOT. 14.0 is ten times more precise than 14. The two numbers have the same value, but they convey different meanings about how trustworthy they are.

Example #3: 2.33 x 6.085 x 2.1. How many significant figures in the answer? Answer - two.

Which number decides this? Answer - the 2.1.

Why?

It has the least number of significant figures in the problem. It is, therefore, the least precise

measurement.

Example #4: (4.52 x 10-4) ÷ (3.980 x 10-6).

How many significant figures in the answer? Answer - three.

Which number decides this? Answer - the 4.52 x 10-4 Why?

It has the least number of significant figures in the problem. It is, therefore, the least precise measurement. Notice it is the 4.52 portion that plays the role of determining significant figures; the exponential portion plays no role.

###### Propagating Errors through Calculations Correctly

In this section we present better rules that are usually better estimation of how good our results are, at the cost of slightly more calculations.

Addition and Subtraction

The square of the error after addition or subtraction is the sum of the squares of the input errors.

Mathematically, we write:

If	S= ∑𝑖  𝑋𝑖

(probable error)2 = (∆S)2 = ∑𝑖 (∆𝑋𝑖)2

Consider this example for addition:

| X | ∆X | (∆X)2 |
| --- | --- | --- |
| 56.2 | 0.3 | 0.09 |
| 20.0 | 0.1 | 0.01 |
| 9.70 | 0.07 | 0.0049 |
| 85.90 | 0.47 | 0.1049 |

Taking the square root of the sum of the errors (0.1049)1/2 =0.32

We find the final result: 85.9 ±0.3 (we take one significant figure in error reading)

The subtraction example proceeds similarly, with the caution that errors always add, even though the quantities may be subtracted.

Multiplication and Division

The square of the relative errors of the product or quotient is the sum of the squares of the relative errors of the individual numbers.

Mathematically, we write:

If	P = ∏𝑖  𝑋𝑖

(Probable relative error)2 = (∆P)2 = ∑  (∆𝑋𝑖)2

𝑃

if X1 = 15.30 and X2 = 41.5

∆X1 = 0..03 and ∆X2 = 0.2

Then,

P= X1 . X2 = 634.95

𝑖	𝑋𝑖

(  ∆P  )2 = ( 0.03 )2 + ( 0.2 )2

634.95

So

∆P = 3.30

15.30

41.5

So the best estimation for the correct answer and uncertainty is 635±3

Powers

The resultant relative error is n times the relative error of the number.

if	P=Xin

(Probable relative error) = (∆P) =n(∆Xi)

𝑃	𝑋𝑖

##### Finding Densities and Uncertainties of the Objects

We have to find the Density and Uncertainty of a Sphere and a ring.

###### For a Sphere

Formula for density of a sphere is: Density=mass/volume

Formula for uncertainty would be:

( ∆𝞀)2 = ( ∆m)2 + ( ∆v)2

𝞀	m	v

Mass could be found through weighing balance. Uncertainty in mass is equal to the half of Least count of the weighing machine

Volume could be found by formula:

V= 4 𝜋( d)3

3	2

Here diameter in unknown, we can find it through Vernier Caliper.

Procedure to Find Diameter of a Sphere using Vernier calipers

- Find the least count of the Vernier caliper

Least Count=	minimum reading on main scale

total number of division on vernier scale

L.C = 1mm

20 div

= 0.05mm

- Find zero error if any

- Place the sphere in between the inner jaws to measure the diameter Note the main scale reading.

- Note the Vernier scale division coinciding with the main scale division and then multiply it with least count to get the Vernier scale reading.

- Add the main scale reading into Vernier scale reading.

- In case of zero error, apply the zero correction to get the total reading of diameter of the sphere.

- Repeat it thrice and take their mean value.

Observations

| No. of Obs. | Quantity to be measured | Main scale reading M (cm) | Vernier scale division coinciding with main scale n (div) | Vernier scale reading V= n x L.C (cm) | Total Reading (cm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Observed T=M+V | Corrected T ± Z.C |
| 1 | Diameter |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

Mean Diameter of Sphere=	cm

Volume and uncertainty

Apply Volume’s formula Vs= 4 𝜋( d)3

3	2

Vs=	cm3

∆Vs = 3 ∆ds/ds

∆ds= half of L.C of Screw gauge =	mm

∆ds=	cm

∆Vs=	cm3

Density of a sphere and its uncertainty

Find the density and uncertainty.

Formula for density of a sphere is: Density=mass/volume

Density=……………………………………. Formula for uncertainty would be:

( ∆𝞀𝑠 )2 = ( ∆m𝑠 )2 + ( ∆V𝑠 )2

𝞀𝑠

m𝑠

V𝑠

∆ ρ𝑠 =………………………………………….

###### For a Ring

Formula for density of a sphere is: Density=mass/volume

Formula for uncertainty would be:

( ∆𝞀)2 = ( ∆m)2 + ( ∆v)2

𝞀	m	v

Mass could be found through weighing balance. Uncertainty in mass is equal to the half of least count of the weighing machine

Volume of a ring with height h, outer diameter D1, and inner diameter D2 is

V= 𝛑 h (D12−D 2)

4	2

Here diameter and height is unknown, we can find it through Vernier Caliper.

Procedure to Find inner and outer diameter of a ring using Vernier caliper

- Find the least count of the Vernier caliper

Least Count=	minimum reading on main scale

total number of division on vernier scale

L.C = 1mm

20 div

= 0.05mm

- Find zero error if any

- Place the ring in between the inner jaws to measure the outer diameter and place the ring around the upper jaws to measure the inner diameter

- Note the main scale reading.

- Note the Vernier scale division coinciding with the main scale division and then multiply it with least count to get the Vernier scale reading.

- Add the main scale reading into Vernier scale reading.

- In case of zero error, apply the zero correction to get the total reading of diameter of the sphere.

- Repeat it thrice and take their mean value.

Observations

| No. of Obs. | Quantity to be measured | Main scale reading M (cm) | Vernier scale division coinciding with main scale n (div) | Vernier scale reading V= n x L.C (cm) | Total Reading (cm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Observed T=M+V | Corrected T ± Z.C |
| 1 | Inner Diameter |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 1 | Outer Diameter |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

Mean Inner Diameter of ring=	cm

Mean Outer Diameter of ring=	cm

Procedure to Find the height of a ring using Screw gauge

- Find the least count of the screw gauge

Least Count=	pitch of the screw

total number of division on circular scale

L.C = 0.5 mm = 0.01mm

50 div

- Find zero error if any

- Place the ring height wise in between the studs and rotate the milled head slowly until ring is

pressed gently.

- Note the linear scale reading.

- Note the circular scale division coinciding with the pitch line and then multiply it with least count to get the circular scale reading.

- Add the main scale reading into circular scale reading.

- In case of zero error, apply the zero correction to get the total reading of height of the ring.

- Repeat it thrice and take their mean value.

Observation

| No. of Obs. | Quantity to be measured | Linear scale reading M (mm) | Circular scale division coinciding with pitch line n(div) | Circular scale reading V= n x L.C (mm) | Total Reading (mm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Observed T=M+V | Corrected T ± Z.C |
| 1 | Height of ring |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

Mean height of ring=	mm

Mean height of ring=	cm

Volume and uncertainty

Vr= 𝛑 h (D12−D 2)

4	2

Vr=……………………………………………. Vr=…………………………………………….

Formula for uncertainty in determination of volume of a ring:

( ∆V𝑟 )2 = ( ∆h )2 + ( 2D1∆D1 )2 + ( 2D2∆D2 )2

V𝑟

h	𝐷12−𝐷22

𝐷12−𝐷22

∆V𝑟 =…………………………………………

∆V𝑟 =…………………………………………

Density of a ring and its uncertainty

Formula for density of a ring is: Density=mass/volume

Density=……………………………………. Formula for uncertainty in density would be:

( ∆𝞀𝑟 )2 = ( ∆m𝑟 )2 + ( ∆V𝑟 )2

𝞀𝑟

m𝑟

V𝑟

∆ ρ𝑟 =………………………………………….

Observations:	(2 Marks)

##### Review Questions

- What is meant by Least count? Write down the least count of Vernier calipers, screw gauge and weighting balance                                                           (1 Marks)

- When is zero error negative in screw gauge?                                          (1 Marks)

- How many significant figures we take while calculating uncertainties?(1 Marks)

- What are the rules of addition and subtraction regarding significant figures?(1 Marks)

- Explain what zero error is in measurement instruments.

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Explain the rules for rounding off numbers in experimental calculations.

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Round the following to the correct significant figures:

- 62.5347 → 4 significant figures

- 3.78721 → 3 significant figures

- 24.8514 → 3 significant figures

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Write the formula for the density of a sphere. Also write the formula for the uncertainty in density of a sphere

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________